# INTERNSHIP PROJECT REPORT

## BATTERY MANAGEMENT SYSTEM

ELEVANCESKILLS TECHNOLOGY

NAME: VAIBHAVI BH

INTERN ID : NZokw1

Gmail: vaibhavi.bh21@gmail.com

GitHub link: https://github.com/Vaibhavi9160/ESP32-Battery-Management-System

Battery Management System Using ESP32 and Blynk Cloud

1. Introduction

The Battery Management System (BMS) project was developed as part of the Embedded Systems and IoT internship program. The objective of the project was to design and implement a smart battery monitoring and protection system capable of monitoring battery parameters, detecting faults, providing user interaction through an LCD interface, and enabling cloud-based monitoring using IoT technologies.

The project was implemented using the ESP32 microcontroller, Blynk Cloud Platform, and Wokwi simulation environment. Throughout the internship, the project evolved from basic voltage monitoring to an advanced cloud-connected Battery Intelligence Dashboard.

2. Background

Battery Management Systems play a critical role in modern energy storage applications such as electric vehicles, renewable energy systems, portable electronics, and industrial battery packs. Improper battery operation can lead to performance degradation, reduced lifespan, or safety hazards.

To address these challenges, Battery Management Systems continuously monitor battery health, detect abnormal conditions, and provide protection mechanisms. The increasing adoption of IoT technologies has further enhanced BMS capabilities by enabling remote monitoring, cloud analytics, and intelligent diagnostics.

This project focuses on implementing these concepts using embedded systems and IoT technologies.

3. Learning Objectives

The primary learning objectives of the internship project were:

- Understand battery monitoring and protection principles.

- Gain practical experience with ESP32 microcontrollers.

- Learn analog sensor interfacing and ADC data acquisition.

- Develop fault detection and safety logic.

- Design Human Machine Interfaces (HMI) using LCD displays.

- Implement cloud communication using Blynk IoT.

- Develop real-time monitoring dashboards.

- Improve programming, debugging, and problem-solving skills.

- Gain experience in embedded system development workflows.

4. Activities and Tasks

Task 1: Battery Voltage Monitoring

- Read battery cell voltages using ESP32 ADC channels.

- Perform voltage calculations and monitoring.

- Display battery data through serial communication.

Task 2: Battery Safety Monitoring

- Implement weak cell detection.

- Implement overvoltage detection.

- Monitor battery imbalance conditions.

- Trigger protective actions when faults occur.

Task 3: Human Machine Interface (HMI)

- Integrate 16x2 I2C LCD display.

- Create multiple display screens.

- Display battery information, analytics, and diagnostics.

Task 4: Advanced Fault Management

- Implement rapid voltage fluctuation detection.

- Design recovery state management.

- Implement fault logging and fault handling.

- Activate relay and buzzer during fault conditions.

Task 5: Cloud Telemetry Architecture

- Connect ESP32 to WiFi.

- Integrate Blynk Cloud Platform.

- Send telemetry events and battery data to the cloud.

- Implement cloud connectivity monitoring.

Task 6: Executive Battery Intelligence Dashboard

- Create cloud-based monitoring dashboard.

- Display live battery voltages.

- Visualize historical voltage trends.

- Monitor WiFi RSSI.

- Provide diagnostics and recommendations.

- Implement risk assessment and operational analytics.

5. Skills and Competencies Developed

During the internship, the following technical and professional skills were developed:

Technical Skills

- Embedded C/C++ Programming

- ESP32 Development

- PlatformIO Development Environment

- IoT System Design

- Cloud Telemetry Integration

- Blynk Dashboard Development

- Analog Signal Processing

- Fault Detection Algorithms

- LCD Interfacing

- Git and GitHub Version Control

Professional Skills

- Problem Solving

- Debugging and Troubleshooting

- Project Documentation

- System Design Thinking

- Independent Learning

- Technical Communication

6. Feedback and Evidence

Evidence of successful project implementation includes:

- Functional ESP32 Battery Management System.

- Real-time battery voltage monitoring.

- LCD-based Human Machine Interface.

- Cloud connectivity through Blynk IoT Platform.

- Executive Battery Intelligence Dashboard.

- Historical voltage trend visualization.

- Telemetry event logging.

- GitHub repository containing source code and documentation.

Project screenshots and dashboard outputs demonstrate successful completion of the assigned tasks.

7. Challenges and Solutions

Challenge 1: Cloud Integration Issues

Problem:
Initial difficulties were encountered while configuring Blynk cloud communication and ESP32 connectivity.

Solution:
The cloud architecture was modularized using dedicated cloud management functions and proper Blynk configuration settings.

Challenge 2: Dashboard Data Visualization

Problem:
Some dashboard widgets did not immediately display live data as expected.

Solution:
Different widget configurations were tested, including radial gauges and chart widgets, to ensure effective visualization of battery data.

Challenge 3: Fault Detection Logic

Problem:
Implementing multiple fault detection conditions without conflicts was challenging.

Solution:
Separate fault management and runtime mode logic were introduced to improve system reliability.

Challenge 4: System Debugging

Problem:
Several compilation errors and integration issues occurred during development.

Solution:
Systematic debugging techniques, serial monitoring, and modular code organization were used to identify and resolve issues.

8. Outcomes and Impact

The project successfully achieved its objectives and demonstrated the integration of embedded systems with IoT technologies.

Key outcomes include:

- Development of a complete Battery Management System.

- Real-time battery monitoring and protection.

- Fault detection and diagnostics capabilities.

- Local HMI through LCD display.

- Cloud telemetry and remote monitoring.

- Executive dashboard for battery intelligence visualization.

- Improved understanding of embedded systems and IoT application development.

The project provides a scalable foundation for future battery monitoring applications in industrial and energy storage environments.

9. Conclusion

The Battery Management System project successfully combined embedded systems, safety monitoring, cloud telemetry, and IoT dashboard technologies into a single integrated solution. Through the completion of Tasks 1 through 6, valuable experience was gained in ESP32 programming, fault management, cloud integration, and system design.

The project demonstrates the practical application of modern Battery Management System concepts and highlights the importance of intelligent monitoring, diagnostics, and cloud connectivity in today's energy management solutions. The internship provided significant hands-on experience and strengthened both technical and professional competencies relevant to embedded systems and IoT engineering.